AuraGuard

Camera Tampering & Quality-Monitoring System

Problem Statement 2 — Live Tampering Detection with SMS Alerting

Project Documentation

Process · Design & Architecture · Acceptance Test Plan · Acceptance Test Cases

| Document type | Process, Design, ATP & ATC documentation |
| --- | --- |
| Scope | PS3 — Real-time camera tampering / quality detection |
| Camera under test | Dahua IP camera — Digital Library, Camera 2 (RTSP) |
| Status | Living document — reflects current build state |
| Version | 1.0 |

# Table of Contents

Table of Contents	1

1. Executive Summary	1

Key facts at a glance	1

2. Problem Statement (PS3)	1

2.1 What PS3 requires the system to monitor	1

2.2 Scoring rubric (100 points)	1

3. Design Philosophy	1

3.1 No external datasets, anywhere	1

3.2 No deep-learning training	1

3.3 Explainability over black-box scores	1

3.4 Baseline-and-deviation, not fixed thresholds	1

3.5 Every fix is measured, not guessed	1

4. System Design & Architecture	1

4.1 How the system “learns” a camera	1

4.2 Detection contract	1

4.3 End-to-end pipeline	1

4.4 Physical detectors — what each one does and why	1

Frozen frame  (frozen.py)	1

Camera cover / blockage  (blockage.py)	1

Blur  (blur.py)	1

Camera shake / movement  (movement.py)	1

Glare / flashlight  (glare.py)	1

4.5 Codebase structure	1

5. Process: Build Status by Phase	1

Phase 0 — Ingestion (capture/)	1

Phase 1 — Physical detectors (detect/)	1

Phase 2–3 — Evidence layer & fusion	1

Phase 4 — Dashboard	1

Phase 5 — Alerts	1

6. Known Open Items & Priority Roadmap	1

7. Acceptance Test Plan (ATP)	1

7.1 Purpose	1

7.2 Scope	1

7.3 Test Environment	1

7.4 Entry Criteria	1

7.5 Exit Criteria	1

7.6 Test Levels	1

7.7 Roles	1

7.8 Risks & Mitigations	1

8. Acceptance Test Cases (ATC)	1

8.1 Ingestion & Calibration	1

8.2 Physical Anomaly Detectors	1

8.3 Position, Angle & Field of View	1

8.4 Confirmation, Fusion & Alerting	1

8.5 Dashboard & Real-Camera Integration	1

9. Glossary	1

# 1. Executive Summary

AuraGuard is a camera tampering and video-quality monitoring system. It watches a live camera feed, learns what “normal” looks like for that specific camera during a short on-site calibration window, and raises an SMS alert — with a plain-language reason — when the feed's position, sharpness, brightness, color, or physical state deviates from that learned normal for long enough to rule out a passing shadow or a bump.

The project was rescoped from a broader “Live Tampering Detection” concept into Problem Statement 3 (PS3) once the venue rubric was published. PS3 marks on five criteria — real-time detection with SMS alerts, building the training dataset from live video itself, coding efficiency, understanding of IP camera functional settings, and prototype feasibility — and the build has been re-prioritized around exactly those five, in that order of point value.

This document is the project's documentation package: how the system is built and why (Process & Design), what the architecture looks like end to end (Design), and how it is being verified against PS3's requirements (Acceptance Test Plan and Acceptance Test Cases). It reflects the system's current, real build state — including the parts still open — rather than only the finished parts.

### Key facts at a glance

- Runs against a real Dahua IP camera over RTSP (172.16.10.235:554, channel 1) — not a mock feed.

- No external or labelled training data anywhere in the pipeline: every camera calibrates itself from its own first seconds of video.

- Classical computer vision throughout (SSIM, optical flow, Laplacian variance, histograms, FFT, ORB keypoints + RANSAC) — no GPU, no inference server, no cloud round-trip, with one narrow exception (a pretrained YOLO model used as-is for a now-deprioritized screen-replay check).

- Five physical-layer detectors built and substantially verified: frozen frame, camera cover, blur, camera shake/movement, and glare/flashlight.

- Three PS3-required detectors are outstanding: camera color / white-balance shift, position/angle-in-degrees, and field-of-view (zoom/framing).

- SMS alerting — worth 30 of PS3's 100 points — is not yet built and is the single highest-priority gap.

# 2. Problem Statement (PS3)

Problem Statement 3 asks for a system that monitors live video camera feeds and generates SMS alerts when video quality is deliberately or naturally affected. The stated motivation: human operators watching screens for extended shifts miss gradual degradation, and recording systems keep recording even after quality has silently dropped — so the footage is only discovered to be useless when it is finally needed.

### 2.1 What PS3 requires the system to monitor

- Position — camera angle drift, expressed as length & height deviation in degrees.

- Software anomalies — blur, camera color shift, sharpness loss, brightness shift, frozen frames.

- Physical anomalies — camera cover / obstruction.

### 2.2 Scoring rubric (100 points)

| Criterion | Points | What it rewards |
| --- | --- | --- |
| Real-time detection + SMS alerts | 30 | Detection latency and a working SMS delivery path, not just a console log |
| Creating a training dataset from live video | 30 | Self-calibration from the live feed itself — no external/labelled dataset |
| Coding efficiency | 20 | Clean, non-redundant, maintainable implementation |
| Understanding IP camera functional settings | 10 | Correct handling of RTSP streams, sub-streams, camera-side parameters |
| Feasibility of prototype | 10 | Whether the thing actually runs against real hardware, not just synthetic clips |

Two rubric lines shape the whole architecture directly: the 30-point “training dataset from live video” line is why the system deliberately avoids external datasets and instead builds an AdaptiveBaseline from the camera's own footage (see §4.1), and the 30-point SMS line is why SMS delivery — not Telegram, the original plan — is treated as a hard requirement rather than a nice-to-have.

# 3. Design Philosophy

Five principles have been carried through the entire build, and most of the architectural decisions below trace back to one of them.

### 3.1 No external datasets, anywhere

The organisers were explicit that the system should learn “normal” from real live video, not from labelled external data. This also happens to be exactly what PS3's 30-point training-dataset criterion is asking for: self-calibration, not supervised machine learning.

### 3.2 No deep-learning training

Everything in the active detection path is classical signal processing: SSIM, optical flow, Laplacian variance, histogram analysis, FFT, and ORB keypoint matching with RANSAC homography estimation. The one exception is a pretrained YOLO object detector used as-is (zero fine-tuning) for an earlier screen-replay check, which has since been deprioritized out of the PS3 demo path.

### 3.3 Explainability over black-box scores

Every detector signal carries a human-readable evidence string — for example “SSIM 0.998, noise floor collapsed, held for 38 frames” — rather than a bare confidence number. The intent is that a technician reading an alert at 2 a.m. knows what to bring before leaving the desk.

### 3.4 Baseline-and-deviation, not fixed thresholds

A shared AdaptiveBaseline object learns what is normal for the specific camera in use — brightness, sharpness, edge density, histogram shape, noise floor — during a short calibration window. Every detector then measures deviation from that learned baseline, never a hardcoded global threshold.

### 3.5 Every fix is measured, not guessed

The build follows one repeated pattern for every real bug encountered: diagnose the actual cause with logged numbers, build a synthetic reproduction to confirm the hypothesis, fix the real mechanism, then re-verify against real hardware. Proposed fixes that didn't hold up against evidence have been rejected rather than shipped on faith — the movement-detector “coherence” explanation (§4.4) is a documented example of this discipline catching itself out.

# 4. System Design & Architecture

## 4.1 How the system “learns” a camera

On startup (or restart at a new site), AdaptiveBaseline runs a short calibration window against the live feed. It builds a true running mean of brightness, contrast, Laplacian variance (sharpness), edge density, and histogram entropy/shape for roughly the first two seconds, then switches to a slow exponential moving average so the baseline can track very gradual, legitimate change (e.g. dusk) without being fooled by a sudden tamper event.

Two safeguards matter here. First, baseline updates freeze during an active alert, so a covered lens cannot be slowly “learned” as the new normal. Second, the noise-floor tracker is asymmetric — it falls slowly and rises far more slowly — which is what lets the frozen-frame detector tell a genuinely still subject apart from a genuinely frozen feed (§4.4).

## 4.2 Detection contract

Every detector, regardless of what it measures, returns the same Signal shape. This is enforced at construction — not just documented — so a detector that returns an out-of-range confidence, empty evidence, or a malformed bounding box never reaches the fusion layer.

| Field | Type | Purpose |
| --- | --- | --- |
| signal_name | str | Which detector fired |
| triggered | bool | Whether this frame is anomalous for this signal |
| confidence | float (0–1) | How sure the detector is |
| evidence | str | Human-readable reason, e.g. “sharpness at 38% of baseline, held 5.2s” |
| bbox | list ¦ None | Optional region of interest |
| pool | str | Physical / Digital / Quality — which trust pool this signal belongs to |

Because every detector — present or future — speaks this same contract, the fusion layer never needs to know which detector produced a signal. Adding a fifth or sixth detector is a drop-in, not a rewrite of anything upstream.

## 4.3 End-to-end pipeline

The diagram below traces one frame from capture through to an alert. It is a direct text rendering of the current architecture, not a screenshot.

Camera / RTSP / Webcam Feed
              |
              v
      Shared Preprocessing
  resize . normalize . ring buffer (30 frames)
              |
              v
          Evidence Layer                        <- runs on every frame,
  hash(frame + previous_hash)                       regardless of verdict
     -> tamper-evident chain
              |
      --------+--------
      |               |
      v               v
Branch A                        Branch B
Temporal Integrity              Spatial Authenticity
(every frame)                   (on suspicion only)
 
SSIM tracking                   YOLO26 - screen/device detection (replay attack)
Optical flow                    Moire pattern (FFT)
Frame differencing              Bezel / edge detection
Histogram correlation
pHash autocorrelation
-> freeze / loop / replay /     -> screen-replay / overlay injection
   blockage signals
 
      |               |
      ----+------------
           |
           v
        Confidence Fusion
  weighted rule engine (not ML)
  verdict . type . confidence . named evidence signal
           |
           v
   Verified Against Hash Chain
  (was this frame's continuity intact when the verdict fired)
           |
           v
      Live Dashboard / Alert
  (bounding boxes, confidence, named signal, chain status)

Figure 1 — Full pipeline: ingestion → tamper-evident hash chain → temporal-integrity and spatial-authenticity branches → confidence fusion → chain verification → dashboard / alert.

For the PS3 demo path specifically, the flow is trimmed to: Camera/RTSP source → frame ingestion → adaptive baseline (this is the “training dataset” the rubric asks for) → physical trust worker pool → trust fusion → event correlation → live dashboard + SMS alert manager. The hash-chain, replay-detection, and screen-authenticity work shown in Figure 1 (Branch B) is built and functional but explicitly out of scope for PS3's rubric — PS3 scores camera-settings deviation, not feed provenance — and is kept in the codebase as a documented future roadmap rather than deleted.

## 4.4 Physical detectors — what each one does and why

### Frozen frame  (frozen.py)

Distinguishes a genuinely frozen feed from a still subject (a person standing still is not a frozen camera). Uses changed-pixel ratio measured against the learned noise floor, rather than relying on SSIM alone.

Verification status: Proven: the liveness ratio holds at 1.00× across a 20× range of camera noise levels.

### Camera cover / blockage  (blockage.py)

Detects a physically obstructed lens. An earlier version required a brightness collapse, which never happens if the room was already dark. Fixed using hist_distance — histogram shape, which is brightness-independent — an already-computed but previously unused metric.

Verification status: hist_distance gives 3.5× separation versus the metrics that were failing. The dark-room boundary threshold is still pending validation on real footage.

### Blur  (blur.py)

Laplacian variance measured against baseline. Had a real cross-trigger bug: torch/glare washing out local contrast collapsed the variance and was misread as blur. Fixed with a shared exposure-compensation signal that suppresses the blur signal when glare's signature is present.

Verification status: Fixed and cross-trigger-tested against the glare detector.

### Camera shake / movement  (movement.py)

Detects camera-mount movement. An earlier “coherence” explanation for its own logic was tested and found wrong — a small moving object scored as high coherence as genuine shake. Corrected to the real discriminators: median flow magnitude plus moving-pixel fraction.

Verification status: Corrected after synthetic reproduction disproved the original hypothesis.

### Glare / flashlight  (glare.py)

Detects a light deliberately aimed at the lens. Auto-exposure compensation keeps pixels below the raw overexposure clip threshold, so naive overexposure detection structurally cannot fire. Fixed by detecting the compensation itself: a bright spot plus the rest of the scene darkening plus global brightness held artificially steady.

Verification status: Fixed; root cause was auto-exposure, not detector sensitivity.

## 4.5 Codebase structure

The repository separates ingestion, detection, source resolution, synthetic test tooling, and a clearly-labelled archive of work that exists but is not wired into the live pipeline:

Aura_Guard/
+-- capture/                     grabber, frame, stats
+-- detect/                      contract, baseline, config, metrics, scoring,
|                                 calibration, engine, exposure, occlusion,
|                                 absolute_cover, absolute_brightness, detectors/
+-- camera_sources.py            ipcam / ipcam:sub URL resolver
+-- ip_camera.py                 camera credentials, RTSP URL builder
+-- requirements.txt
+-- sample.mp4                   Phase 0 fixture (actively used by 4+ scripts)
+-- make_sample_video.py         generates sample.mp4
+-- staged_scenes.py             synthetic event generator
+-- test_capture.py              Phase 0 acceptance test
+-- test_detectors_synthetic.py  full regression matrix (synthetic)
+-- test_detectors.py            live full-detector staged test
+-- test_optics.py               live cover+blur+brightness staged test
+-- test_cover.py                live cover-only staged test
+-- check_cover.py               one-shot absolute cover check
+-- measure_cells.py             per-cell grid diagnostic
+-- verify_freeze.py             frozen-frame live/frozen separation test
|
+-- archive/                     not deleted, not wired in
    +-- diagnose_metrics.py      superseded by check_against_live + measure_cells.py
    +-- compare_streams.py       FOV investigation, scoped out
    +-- fov_check/               stale images from that investigation
    +-- cover_phase/             frozen snapshot, predates blur/brightness/occlusion
    +-- Positioning/             self-contained detector, never imported anywhere,
    |                            not wired into engine.py
    +-- normal_sample.mp4        never referenced by any script
    +-- normal_sample2.mp4       never referenced by any script

# 5. Process: Build Status by Phase

The build proceeds in phases matching the pipeline stages. Status below is honest about what is verified versus what is built-but-pending versus what has not started — this section is the single source of truth for “what actually works today.”

### Phase 0 — Ingestion (capture/)

Status: Built & verified

- Threaded FrameGrabber: producer/consumer with a bounded queue.

- Frame dataclass carries sequence number, time.monotonic() timestamp (authoritative), wall clock (for logs), and source index.

- Drop policy is ‘oldest’ by default; drops are never silent (rate-limited warnings + summary) and gaps are detectable two independent ways.

- File sources are paced to native FPS; camera sources are unpaced.

- Verified stable at 30fps on both a synthetic clip and a real webcam, zero drops on clean runs; a forced-failure test confirmed the drop accounting balances exactly.

### Phase 1 — Physical detectors (detect/)

Status: Built & mostly verified; real-camera fixes ongoing

- Shared contract.py and baseline.py described in §4.1–4.2.

- Five detectors built: frozen, blockage, blur, movement, glare (see §4.4 for detail on each).

- Open item: dark-room cover/blur boundary threshold is real-camera-data pending, last checked only against synthetic data.

- Open item: noise-floor drift observed on long real sessions (+53.8% over a session) — reported, not yet fixed, not currently dangerous.

- Open item: a separate ip_camera.py script is confirmed working against the real venue camera, but is not yet confirmed merged into capture/grabber.py as the live source for the full detector pipeline.

- Not yet built (needed specifically for PS3): camera color / white-balance detector, position/angle-in-degrees detector (in progress in a separate session), field-of-view detector.

### Phase 2–3 — Evidence layer & fusion

Status: Built for original scope, now demoted

- Hash chain / evidence layer, loop/replay detection, and screen-replay (YOLO phone/TV) detection are all built and working.

- Explicitly out of scope for PS3's rubric — PS3 scores camera-settings deviation, not feed authenticity or record provenance. Kept as a documented future roadmap, not deleted from the codebase.

- The rule-based, confidence-weighted fusion engine exists; it still needs to be confirmed wired to whichever new detectors are added for PS3 (color, position, field-of-view).

### Phase 4 — Dashboard

Status: Built; needs a PS3-specific addition

- FastAPI + WebSocket live view, plus an upload/analysis mode, exist from earlier phases.

- Still needed: a visible calibration/training-status indicator, since baseline calibration — which is what satisfies PS3's 30-point ‘training dataset’ line — currently runs invisibly to anyone watching the dashboard.

### Phase 5 — Alerts

Status: Not built — highest-priority gap

- Original plan was Telegram; PS3 specifically requires SMS. Nothing built yet.

- This single item represents 30 of PS3's 100 points, currently scoring zero.

# 6. Known Open Items & Priority Roadmap

In order, reflecting both point value and dependency order — later items build on earlier ones (SMS alerting, for instance, needs the new detectors' signals to have something worth alerting on):

| # | Item | State |
| --- | --- | --- |
| 1 | Confirm / finish real IP camera (RTSP) integration into the main capture pipeline | In progress |
| 2 | Build and merge the position/angle-in-degrees detector | In progress (separate session) |
| 3 | Build the camera-color (white-balance shift) detector | Not started |
| 4 | Build the field-of-view (zoom/framing) detector | Not started |
| 5 | Build SMS alerting — the single biggest point gap (30/100) | Not started |
| 6 | Add a visible calibration-status indicator to the dashboard | Not started |
| 7 | Real-camera validation pass on everything above | Not started — required; this codebase has repeatedly found real bugs synthetic testing alone missed |

The recurring lesson driving item 7's priority: several real bugs in the physical detectors (the dark-room cover false-negative, the glare/blur cross-trigger, the movement-detector's wrong self-explanation) were invisible in synthetic testing and only surfaced against the real camera. Every new detector built for PS3 should expect the same and budget time for a real-camera validation pass, not just a synthetic one.

# 7. Acceptance Test Plan (ATP)

## 7.1 Purpose

This ATP defines how AuraGuard will be verified against PS3's requirements before demo/submission. It covers what is tested, in what environment, with what data, by whom, and what “done” means for each PS3 scoring criterion.

## 7.2 Scope

In scope: all five PS3-relevant physical/software anomaly detectors (frozen frame, camera cover, blur, camera shake, glare, plus the three still being built — color shift, position/angle, field-of-view), the adaptive-baseline self-calibration mechanism, RTSP ingestion against the real venue camera, and SMS alert delivery end to end.

Out of scope for this ATP: the hash-chain/evidence layer, loop/replay detection, and screen-replay (YOLO) detection. These are built and functional but explicitly outside PS3's rubric, and are covered only informally as “future roadmap,” not against formal acceptance criteria.

## 7.3 Test Environment

| Item | Detail |
| --- | --- |
| Primary camera | Dahua IP camera, Digital Library / Camera 2, RTSP 172.16.10.235:554, channel=1 |
| Secondary source | USB webcam and/or sample.mp4 file source, for tests that need a controlled, repeatable feed |
| Network | Venue LAN; sub-stream (channel=1 subtype=1 equivalent) available for bandwidth-constrained tests |
| Synthetic tooling | staged_scenes.py (synthetic event generator), make_sample_video.py, test_detectors_synthetic.py |
| SMS delivery | To be defined once the SMS provider/gateway is selected — currently unbuilt |

## 7.4 Entry Criteria

- The detector or feature under test has a merged, contract-compliant implementation (returns a valid Signal per §4.2).

- A calibration window can complete against the test source without error.

- For real-camera test cases: the venue camera is reachable over RTSP at test time.

## 7.5 Exit Criteria

- All Priority-1 acceptance test cases in §8 pass against the real camera, not only against synthetic clips.

- Every triggered alert carries a non-empty, specific evidence string (per §3.3) — a bare true/false trigger is treated as a failure even if the underlying detection was correct.

- No detector produces a sustained false alarm against at least 10 minutes of undisturbed real footage per PS3 anomaly type.

- SMS delivery is confirmed end to end at least once against a real phone number before sign-off.

## 7.6 Test Levels

- Unit / synthetic — test_detectors_synthetic.py and staged_scenes.py, run first for fast iteration; catches logic errors cheaply.

- Live staged — test_detectors.py, test_optics.py, test_cover.py, verify_freeze.py: real camera, deliberately staged anomalies (hand over lens, torch, physical nudge).

- End-to-end acceptance — the full pipeline against the real Dahua camera, unstaged, over an extended run, checked against the ATCs in §8.

## 7.7 Roles

| Role | Responsibility |
| --- | --- |
| Detector owner | Implements and unit/synthetic-tests their detector against the shared contract |
| Integration owner | Confirms RTSP ingestion, fusion wiring, and dashboard/alert integration for each new detector |
| Test executor | Runs the ATCs in §8 against real hardware and records pass/fail with evidence strings |

## 7.8 Risks & Mitigations

| Risk | Mitigation |
| --- | --- |
| Synthetic-only testing hides real bugs (established pattern in this codebase) | Every ATC in §8 has a real-camera variant; synthetic-only pass is not sufficient for exit |
| SMS gateway not yet chosen — 30-point criterion currently at zero | Flagged as the top roadmap priority (§6); ATP treats end-to-end SMS delivery as a hard exit criterion |
| Three PS3-required detectors (color, position, FOV) still incomplete close to deadline | ATCs for these are written now against the contract so integration isn't blocked on them being finished |
| Noise-floor drift over long sessions could eventually cause false negatives | Tracked as a monitored, non-blocking risk (§6); long-session ATC included to catch regression |

# 8. Acceptance Test Cases (ATC)

Status legend: [READY] detector built — test can run today.  [PARTIAL] built but with a known open issue affecting this case.  [BLOCKED] feature not yet built — test case defined in advance.

## 8.1 Ingestion & Calibration

| TC ID | Detector / Feature | Test Case & Steps | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-01 | FrameGrabber (capture/) | 1. Start ingestion from the real Dahua RTSP source. 2. Run for 10 minutes under normal network conditions. 3. Compare producer frame count to consumer sequence count. | Zero unaccounted frame drops; any drop is logged with a rate-limited warning, never silent. | [READY] |
| TC-02 | FrameGrabber — forced failure | 1. Deliberately stall the consumer to force queue overflow. 2. Restore normal consumption. 3. Inspect drop accounting. | Drop policy applies (oldest dropped first); producer-count and consumer-sequence gap detection agree exactly. | [READY] |
| TC-03 | AdaptiveBaseline calibration | 1. Start calibration against the real camera from a cold start. 2. Observe brightness, sharpness, edge-density, and histogram-shape values during the ~2s running-mean window. 3. Confirm transition to slow EMA tracking after warmup. | Baseline values stabilize within the calibration window and track only slow, legitimate drift afterward (not sudden events). | [READY] |
| TC-04 | AdaptiveBaseline — freeze during alert | 1. Trigger any detector alert (e.g. cover the lens). 2. While the alert is active, continue feeding the anomalous frames. 3. Check whether baseline values shift toward the anomalous state. | Baseline does not update while an alert is active — the anomaly is never learned as the new normal. | [READY] |

## 8.2 Physical Anomaly Detectors

| TC ID | Detector / Feature | Test Case & Steps | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-05 | Frozen frame | 1. Calibrate against a live scene with a person standing still. 2. Confirm no false trigger. 3. Physically freeze/disconnect the feed (or substitute a static image). | No trigger while the subject is merely still; a clear trigger, with evidence citing the changed-pixel ratio against the noise floor, once the feed is genuinely frozen. | [READY] |
| TC-06 | Frozen frame — noise-level range | 1. Repeat TC-05 at three different camera noise levels (e.g. varying gain/low-light settings). 2. Confirm liveness ratio consistency. | Liveness ratio holds at ~1.00× across at least a 20× range of noise levels, per the existing verification. | [READY] |
| TC-07 | Camera cover — lit room | 1. Calibrate in a normally lit room. 2. Cover the lens fully. 3. Observe trigger and evidence. | Trigger fires with evidence referencing hist_distance (histogram-shape) separation, within the expected hold-time window. | [READY] |
| TC-08 | Camera cover — dark room | 1. Calibrate in a deliberately dark room. 2. Cover the lens. 3. Observe whether brightness-independent detection still fires. | Trigger still fires despite no brightness collapse being available as a signal. (Known open item — threshold last validated only on synthetic data.) | [PARTIAL] |
| TC-09 | Blur | 1. Calibrate on a sharp scene. 2. Introduce real defocus (physically adjust focus or use a diffusion filter). 3. Observe trigger and evidence. | Trigger fires with evidence citing Laplacian variance versus baseline. | [READY] |
| TC-10 | Blur — glare cross-trigger | 1. Calibrate on a sharp scene. 2. Introduce strong glare/torch light without defocusing. 3. Confirm blur detector does NOT fire. | Blur signal is suppressed when the exposure-compensation signal indicates glare is present — no false blur trigger. | [READY] |
| TC-11 | Camera shake / movement | 1. Calibrate with the camera mount stable. 2. Physically nudge or shake the mount. 3. Observe trigger and evidence. | Trigger fires, evidence cites median flow magnitude and moving-pixel fraction. | [READY] |
| TC-12 | Camera shake — false-positive rejection | 1. Calibrate with the mount stable. 2. Introduce a small moving object in frame (e.g. a person walking through) without moving the mount. 3. Confirm no shake trigger. | No trigger — small in-frame motion is not misread as mount shake (regression test for the corrected discriminator). | [READY] |
| TC-13 | Glare / flashlight | 1. Calibrate under normal lighting. 2. Aim a bright light source (torch/flashlight) at the lens. 3. Observe trigger and evidence. | Trigger fires with evidence citing the detected auto-exposure compensation pattern (bright spot + rest of scene darkened + brightness held steady). | [READY] |
| TC-14 | Camera color / white-balance shift | 1. Calibrate under a fixed color temperature. 2. Introduce a real color shift (colored gel over lens, or change of light source). 3. Observe trigger and evidence. | Trigger fires with evidence citing measured color/white-balance deviation from baseline. | [BLOCKED] |

## 8.3 Position, Angle & Field of View

| TC ID | Detector / Feature | Test Case & Steps | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-15 | Position/angle drift — detection | 1. Calibrate a reference frame with ORB keypoints. 2. Physically rotate/pan the camera mount by a known angle. 3. Compare reported angle to the physically applied angle. | Reported pan/tilt in degrees is within an acceptable error margin of the known applied rotation. | [BLOCKED] |
| TC-16 | Position/angle drift — false-positive control | 1. Calibrate a reference frame. 2. Apply vibration/bumps without a sustained mount change. 3. Confirm no false angle-drift trigger. | No trigger — transient vibration is rejected the same way other detectors reject transients. | [BLOCKED] |
| TC-17 | Field of view (zoom/framing) | 1. Calibrate a reference frame and field of view. 2. Change zoom or reframe the camera. 3. Observe trigger and evidence. | Trigger fires with evidence describing the change in framing/zoom versus the calibrated reference. | [BLOCKED] |

## 8.4 Confirmation, Fusion & Alerting

| TC ID | Detector / Feature | Test Case & Steps | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-18 | Streak / stability confirmation | 1. Introduce a brief, single-frame anomaly (e.g. one frame of motion blur from a passing object). 2. Confirm no alert fires. 3. Introduce a sustained anomaly of the same type held across multiple frames. | Single-frame transients never escalate to an alert; only anomalies that hold steady across a run of consecutive frames trigger escalation. | [READY] |
| TC-19 | Evidence-string quality | 1. Trigger each built detector in turn. 2. Inspect the evidence field of each resulting Signal. | Every triggered signal has a specific, human-readable evidence string (not a bare code or confidence number) per the contract in §4.2. | [READY] |
| TC-20 | Fusion — multi-detector agreement | 1. Stage an event likely to trip two detectors at once (e.g. a hand briefly covering and shaking the lens). 2. Observe fused output. | Fusion produces one coherent, weighted verdict rather than duplicate or conflicting alerts. | [READY] |
| TC-21 | SMS alert — delivery | 1. Trigger a confirmed anomaly. 2. Observe whether an SMS is sent to a configured test number. 3. Measure time from confirmation to SMS receipt. | SMS is received, contains the evidence string (not just a bare alert code), within an acceptable real-time latency. | [BLOCKED] |
| TC-22 | SMS alert — cooldown / storm suppression | 1. Trigger a sustained anomaly that would otherwise re-fire every frame. 2. Observe SMS frequency over the duration of the event. | A cooldown period prevents repeated SMS delivery for the same ongoing fault; one SMS per distinct event, not one per frame. | [BLOCKED] |

## 8.5 Dashboard & Real-Camera Integration

| TC ID | Detector / Feature | Test Case & Steps | Expected Result | Status |
| --- | --- | --- | --- | --- |
| TC-23 | RTSP integration into main pipeline | 1. Point capture/grabber.py at the real Dahua RTSP URL via camera_sources.py. 2. Confirm frames reach the detector engine (not just the standalone ip_camera.py script). 3. Run for an extended session. | The main detector pipeline consumes live frames directly from the real camera end to end, matching what ip_camera.py already proved works standalone. | [PARTIAL] |
| TC-24 | Calibration-status visibility | 1. Open the live dashboard during a cold-start calibration window. 2. Observe whether calibration progress/state is visible. | A visible indicator shows the system is in ‘training/calibrating’ state and when it transitions to ‘monitoring’ — satisfies PS3's training-dataset criterion being demonstrable, not just present in logs. | [BLOCKED] |
| TC-25 | Sub-stream operation under network pressure | 1. Switch source to the camera's sub-stream (lower bandwidth). 2. Repeat TC-05, TC-07, TC-09, TC-11, TC-13 against the sub-stream. | Detectors continue to function correctly on the sub-stream, confirming the pipeline is not resolution-dependent. | [READY] |
| TC-26 | Long-session baseline stability | 1. Run an extended real-camera session (several hours). 2. Track noise-floor and baseline values over time. 3. Compare against the previously reported +53.8% drift. | Drift is bounded and does not cause a false negative on a staged anomaly introduced late in the session. | [PARTIAL] |
| TC-27 | End-to-end PS3 scenario | 1. Run the full pipeline unstaged against the real camera for an extended period. 2. At an undisclosed point, stage one anomaly of each built type in sequence. 3. Confirm each produces a correctly evidenced alert and, once built, an SMS. | Every staged anomaly is detected, correctly attributed, explained in plain language, and (once SMS is built) delivered — with no false alarms during the undisturbed portions of the run. | [PARTIAL] |

# 9. Glossary

| Term | Meaning |
| --- | --- |
| PS3 | Problem Statement 3 — the specific hackathon rubric this project targets |
| AdaptiveBaseline | Component that learns a camera's normal brightness/sharpness/edge/histogram profile from its own live footage |
| Signal | The shared output contract every detector returns (signal_name, triggered, confidence, evidence, bbox, pool) |
| Trust pool | A group of detectors sharing a category — Physical, Digital, or Quality/Context |
| Fusion | The rule-based engine that merges signals from all trust pools into one verdict |
| Streak / stability check | The requirement that an anomaly hold across consecutive frames before it escalates to an alert |
| Hash chain / evidence layer | Tamper-evident SHA-256 chaining of frames, built but out of scope for PS3's rubric |
| ATP | Acceptance Test Plan — defines scope, environment, and pass/fail criteria for testing |
| ATC | Acceptance Test Case — an individual, executable test with steps and an expected result |